Andover 2024 Drinking Water Report

Making Safe Drinking Water

Your drinking water comes from a groundwater source: eight wells ranging from 262 to 547 feet deep, that draw water from the Mt. Simon, Quaternary Buried Artesian and Tunnel City-Wonewoc aquifers.

Andover works hard to provide you with safe and reliable drinking water that meets federal and state water quality requirements. The purpose of this report is to provide you with information on your drinking water and how to protect our precious water resources.

Contact Steve Weinhold, Public Utilities Manager, at (763) 767-5180 or s.weinhold@andovermn.gov if you have questions about Andover’s drinking water. You can also ask for information about how you can take part in decisions that may affect water quality.

The U.S. Environmental Protection Agency sets safe drinking water standards. These standards limit the amounts of specific contaminants allowed in drinking water. This ensures that tap water is safe to drink for most people. The U.S. Food and Drug Administration regulates the amount of certain contaminants in bottled water. Bottled water must provide the same public health protection as public tap water. Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of

some contaminants. The presence of contaminants does not necessarily indicate that water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the Environmental Protection Agency’s Safe Drinking Water Hotline at 1-800-426-4791.

Andover Monitoring Results

This report contains our monitoring results from January 1 to December 31, 2024.

We work with the Minnesota Department of Health to test drinking water for more than 100

contaminants. It is not unusual to detect contaminants in small amounts. No water supply is ever completely free of contaminants. Drinking water standards protect Minnesotans from substances that may be harmful to their health.

Learn more by visiting the Minnesota Department of Health’s webpage Basics of Monitoring and testing of Drinking Water in Minnesota

(https://www.health.state.mn.us/communities/environment/water/factsheet/sampling.html).

How to Read the Water Quality Data Tables

The tables below show the contaminants we found last year or the most recent time we sampled for that

contaminant. They also show the levels of those contaminants and the Environmental Protection Agency’s

limits. Substances that we tested for but did not find are not included in the tables.

We sample for some contaminants less than once a year because their levels in water are not expected to change from year to year. If we found any of these contaminants the last time we sampled for them, we included them in the tables below with the detection date.

We may have done additional monitoring for contaminants that are not included in the Safe Drinking Water Act. To request a copy of these results, call the Minnesota Department of Health at 651-201-4700 between 8:00 a.m. and 4:30 p.m., Monday through Friday.

Explaining Special Situations for the Highest Result and Average

Some contaminants are monitored regularly throughout the year, and rolling (or moving) annual averages are used to manage compliance. Because of this averaging, there are times where the Range of Detected Test Results for the calendar year is lower than the Highest Average or Highest Single Test Result, because it occurred in the previous calendar year.

Definitions

AL (Action Level): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

EPA: Environmental Protection Agency

MCL (Maximum contaminant level): The highest level of a contaminant that is allowed in drinking water. MCLs are set as close to the MCLGs as feasible using the best available treatment technology.

MCLG (Maximum contaminant level goal): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs allow for a margin of safety.

MRDL (Maximum residual disinfectant level): The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

MRDLG (Maximum residual disinfectant level goal): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

N/A (Not applicable): Does not apply.

pCi/l (picocuries per liter): A measure of radioactivity.

ppt (parts per trillion): One part per trillion is like one drop in one trillion drops of water, or about one drop in an Olympic sized swimming pool. ppt is the same as nanograms per liter (ng/l).

ppb (parts per billion): One part per billion in water is like one drop in one billion drops of water, or about one drop in a swimming pool. ppb is the same as micrograms per liter (μg/l).

ppm (parts per million): One part per million is like one drop in one million drops of water, or about one cup in a swimming pool. ppm is the same as milligrams per liter (mg/l).

PWSID: Public water system identification.

Monitoring Results for Regulated Substances in drinking water

Section one: Lead and Copper tested at customer taps.

For Lead, sampled on October 14, 2022: EPA’s ideal goal is zero parts per billion, action level is 15 parts per billion. Ninety percent of homes were less than 2 parts per billion, zero out of 33 homes had high levels, range of detected results was zero to two parts per billion, no violation. Typical source: corrosion of household plumbing.

For Copper, sampled on October 14, 2022: EPA’s ideal goal is zero parts per million, action level is 1.3 parts per million. Ninety percent of homes were less than 0.37 parts per million, zero out of 33 homes had high levels, range of detected results was 0.05 to 0.47 parts per million, no violation. Typical source: corrosion of household plumbing.

Section two: Inorganic and Organic Contaminants tested in drinking water.

Gross Alpha: EPA goal zero picocuries per liter, limit 15 picocuries per liter, highest single test result 20 picocuries per liter, range zero to 23 picocuries per liter, no violation, source erosion of natural deposits.

Barium sampled April 28, 2023: EPA goal and limit two parts per million, highest result 0.16 parts per million, no violation, sources include discharge of drilling wastes, discharge from metal refineries, and erosion of natural deposits.

Arsenic: EPA goal zero parts per billion, limit 10 parts per billion, highest result 1.71 parts per billion, no violation, sources include erosion of natural deposits, runoff from orchards, and runoff from glass and electronics production wastes.

Combined Radium: EPA goal zero picocuries per liter, limit five picocuries per liter, highest result 11.5 picocuries per liter, range zero to 11.8 picocuries per liter, no violation, source erosion of natural deposits

Combined Radium: During the year, we had a Combined Radium level above the MCL. Our water system has taken the initiative to limit the amount of water being pumped from the source(s) with elevated radiochemical levels. We have signed an Agreement with MDH (therefore no violation was issued) that ensures all residents receive drinking water containing less than the MCL on an annual basis.

Gross Alpha: During the year, we had a gross alpha level above the MCL. Our water system has taken the initiative to limit the amount of water being pumped from the source(s) with elevated radiochemical levels. We have signed an Agreement with MDH (therefore no violation was issued) that ensures all residents receive drinking water containing less than the MCL on an annual basis.

This section covers contaminants related to disinfection and other substances found in drinking water. For disinfection by-products, total trihalomethanes, or TTHMs, have no ideal goal but the EPA limit is eighty parts per billion. The highest result was thirty-five point eight parts per billion, with a range from seventeen point two to thirty-five point eight. There was no violation. These come from by-products of drinking water disinfection. Total haloacetic acids, or HAA, also have no ideal goal, and the EPA limit is sixty parts per billion. The highest result was thirteen parts per billion, with a range from eight point three to thirteen. No violation. These also come from disinfection by-products. Total chlorine has an ideal goal and limit of four parts per million. The highest result was zero point four four parts per million, with a range from zero point one two to zero point six six. No violation. Chlorine is a water additive used to control microbes.

Next, other substances tested in drinking water. Fluoride has an ideal goal and limit of four parts per million. The highest result was zero point six two parts per million, with a range from zero point five seven to zero point six six. No violation. Fluoride comes from erosion of natural deposits and is also added to water to promote strong teeth.”

Potential Health Effects and Corrective Actions (If Applicable)

Fluoride: If your drinking water fluoride levels are below the optimal concentration range of 0.5 to 0.9 ppm, please talk with your dentist about how you can protect your teeth and your family's teeth from tooth decay and cavities.

For more information, visit: MDH Drinking Water Fluoridation (https://www.health.state.mn.us/communities/environment/water/com/fluoride.html). Fluoride is nature's cavity fighter, with small amounts present naturally in many drinking water sources. There is an overwhelming weight of credible, peer-reviewed, scientific evidence that fluoridation reduces tooth decay and cavities in children and adults, even when there is availability of fluoride from other sources, such as fluoride toothpaste and mouth rinses. Since studies show that optimal fluoride levels in drinking water benefit public health, municipal community water systems adjust the level of fluoride in the water to an optimal concentration between 0.5 to 0.9 parts per million (ppm) to protect your teeth. Fluoride levels below 2.0 ppm are not expected to increase the risk of a cosmetic condition known as enamel fluorosis.

Monitoring Results – Unregulated Substances/Emerging Contaminants

In addition to testing drinking water for contaminants regulated under the Safe Drinking Water Act, we sometimes also monitor for contaminants that are not regulated. Unregulated contaminants do not have legal limits for drinking water. MDH, EPA, and other health agencies may have developed comparison values for some of these compounds. Some of these comparison values are based solely on potential health impacts and do not consider our ability to measure contaminants at very low concentrations nor the cost and technology of prevention and/or treatment. These values may be set at levels that are costly, challenging, or impractical for a water system to meet (for example, large-scale treatment technology may not exist for a given contaminant). Sample data are listed along with comparison values in the table below; it is important to note that these comparison values are not enforceable.

Detection alone of a regulated or unregulated contaminant should not cause concern. The significance of a detection should be determined considering current health effects information. We are often still learning about the health effects, so this information can change over time.

A person drinking water with a contaminant at or below the comparison value would be at little to no risk for harmful health effects. If the level of a contaminant is above the comparison value, people of a certain age or with special health conditions-like a fetus, infants, children, elderly, and people with impaired immunity—may need to take extra precautions. We are notifying you of the unregulated/emerging contaminants we have detected as a public education opportunity.

Unregulated contaminant monitoring helps EPA to determine where certain contaminants occur and whether the Agency should consider regulating those contaminants in the future.

▪ More information is available on MDH’s A-Z List of Contaminants in Water (https://www.health.state.mn.us/communities/environment/water/contaminants/index.html)

▪ Fourth Unregulated Contaminant Monitoring Rule (UCMR 4) (https://www.health.state.mn.us/communities/environment/water/com/ucmr4.html)

▪ Fifth Unregulated Contaminant Monitoring Rule (https://www.epa.gov/dwucmr/fifthunregulated-contaminant-monitoring-rule)

▪ EPA has developed a UCMR5 Program Overview Factsheet (https://www.epa.gov/system/files/documents/2022-02/ucmr5-factsheet.pdf) describing UCMR 5 contaminants and standards. In the past year, your drinking water may have tested for additional unregulated contaminants as part of the Fifth Unregulated Contaminant Monitoring Rule (https://www.epa.gov/dwucmr/fifth-unregulatedcontaminant-monitoring-rule) and results are still being processed. The Unregulated Contaminant Monitoring Rule 5 (UCMR 5) Data finder allows people to easily search for, summarize, and download the available UCMR 5 analytical results (https://www.epa.gov/dwucmr/fifth-unregulated-contaminantmonitoring-rule-data-finder).

unregulated or emerging contaminants tested in drinking water.

For sodium, tested in 2023, the comparison value is twenty parts per million. The highest result was forty-one point three parts per million, with a range from three point nine three to forty-one point three. Note that home water softening can increase the level of sodium in your water.

For sulfate, also tested in 2023, the comparison value is five hundred parts per million. The highest result was ten point two parts per million, with a range from zero to ten point two. Lithium has a comparison value of ten parts per billion. The highest result was four point eight parts per billion, with a range from zero to nine point six.

Perfluorobutanoic acid, or PFBA, has a comparison value of seven thousand parts per trillion. The highest result was ten point eight five parts per trillion, with a range from zero to eleven point one.

Perfluorohexanesulfonate, or PFHxS, tested in 2021, has a comparison value of forty-seven parts per trillion. The highest result was zero point eight three parts per trillion. Perfluoropentanoic acid, or PFPeA, also tested in 2021, has no comparison value. The highest result was zero point five six parts per trillion

In early 2024, MDH released new comparison values for two PFAS compounds, PFOA and PFOS. MDH is still evaluating how to apply these comparison values to drinking water systems. Additionally, EPA released final MCLs for PFOA at 4.0 ppt, PFOS at 4.0 ppt, PFHxS at 10 ppt, HFPO-DA (Gen X) at 10 ppt, PFNA at 10 ppt, and a calculated Hazard Index at 1 (unitless) that will become enforceable April 26, 2029. Additional Information on PFAS system results is available at: Interactive Dashboard for PFAS

Testing in Drinking Water - MN Dept. of Health https://www.health.state.mn.us/communities/environment/water/pfasmap.html. Some People Are More Vulnerable to Contaminants in Drinking Water Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. The developing fetus and therefore pregnant women may also be more vulnerable to contaminants in drinking water. These people or their caregivers should seek advice about drinking water from their health care providers. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline at 1-800-426-4791.

Learn More about Your Drinking Water

Drinking Water Sources

Groundwater supplies 75 percent of Minnesota’s drinking water, and found in aquifers beneath the surface of the land. Surface water supplies 25 percent of Minnesota’s drinking water, and is the water in lakes, rivers, and streams above the surface of the land. Contaminants can get in drinking water sources from the natural environment and from people’s daily activities. There are five main types of contaminants in drinking water sources.

▪ Microbial contaminants, such as viruses, bacteria, and parasites. Sources include sewage treatment plants, septic systems, agricultural livestock operations, pets, and wildlife.

▪ Inorganic contaminants include salts and metals from natural sources (e.g. rock and soil), oil and gas production, mining and farming operations, urban stormwater runoff, and wastewater discharges.

▪ Pesticides and herbicides are chemicals used to reduce or kill unwanted plants and pests. Sources include agriculture, urban stormwater runoff, and commercial and residential properties.

▪ Organic chemical contaminants include synthetic and volatile organic compounds. Sources include industrial processes and petroleum production, gas stations, urban stormwater runoff, and septic systems.

▪ Radioactive contaminants such as radium, thorium, and uranium isotopes come from natural sources (e.g. radon gas from soils and rock), mining operations, and oil and gas production. The Minnesota Department of Health provides information about your drinking water source(s) in a source water assessment, including:

▪ How Andover is protecting your drinking water source(s);

▪ Nearby threats to your drinking water sources;

▪ How easily water and pollution can move from the surface of the land into drinking water sources, based on natural geology and the way wells are constructed. Find your source water assessment at Source Water Assessments (https://www.health.state.mn.us/communities/environment/water/swp/swa) or call 651-201-4700 between 8:00 a.m. and 4:30 p.m., Monday through Friday. Lead in Drinking Water Lead can cause serious health problems, babies, children under six years, and pregnant women are at the highest risk. You may be in contact with lead through paint, water, dust, soil, food, hobbies, or your job. There is no safe level of lead. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. Our water system is responsible for proving high quality drinking water and removing lead pipes from service lines but cannot control the variety of materials used in plumbing components in your home. You can take responsibility by identifying and removing lead materials within your home plumbing and taking steps to reduce your family’s risk. Read below to learn how you can protect yourself from lead in drinking water.

Let the water run before drinking tap water flush your pipes for several minutes by running your tap. If you have a lead service line, you may need to let the water run longer. A service line is the underground pipe that brings water from the main water pipe under the street to your home. Activities such as taking a shower, doing laundry or dishes help keep water moving in your home system but are not a replacement for running the tap before you drink if it has not been used for a long period of time. The only way to know if lead has been reduced by letting it run is to check with a test. If letting the water run does not reduce lead, consider other options to reduce your exposure.  Know your service line materials by contacting your public water system, or you can search for your address online at the Minnesota Lead Inventory Tracking Tool (https://maps.umn.edu/LSL/).

Protect Your Tap: A quick check for lead (https://www.epa.gov/ground-water-and-drinkingwater/protect-your-tap-quick-check-lead) is EPA's step by step guide to learn how to find lead pipes in your home. Use cold water for drinking, making food, and making baby formula. Hot water releases more lead from pipes than cold water. Test your water. In most cases, letting the water run and using cold water for drinking and cooking should keep lead levels low in your drinking water. If you are still concerned about lead, arrange with a laboratory to test your tap water. Testing your water is important if young children or pregnant women drink your tap water. Contact a Minnesota Department of Health accredited laboratory to purchase a sample container and instructions on how to submit a sample: Environmental Laboratory Accreditation Program

(https://eldo.web.health.state.mn.us/public/accreditedlabs/labsearch.seam)

The Minnesota Department of Health can help you understand your test results. Treat your water if a test shows your water has high levels of lead after you let the water run. You can use a filter certified with ANSI/NSF standards 53 and 42 for lead reduction. Read about water treatment units: Point-of-Use Water Treatment Units for Lead Reduction (https://www.health.state.mn.us/communities/environment/water/factsheet/poulead.html)

Information on lead in drinking water, testing methods, and other steps you can take to minimize exposure are available at: Visit EPA Basic Information about Lead in Drinking Water (http://www.epa.gov/safewater/lead) Visit the Minnesota department of Health Lead in Drinking Water (https://www.health.state.mn.us/communities/environment/water/contaminants/lead.html)

To learn about how to reduce your contact with lead from sources other than your drinking water, visit Lead Poisoning Prevention: Common Sources (https://www.health.state.mn.us/communities/environment/lead/fs/common.html)

Be Aware: Head Start Programs, Child Care Centers, Public and Charter Schools all have requirements to test for lead in drinking water. These programs can learn more about requirements and resources for testing and remediation at MDH Drinking Water in Schools and Child Cares (https://www.web.health.state.mn.us/communities/environment/water/schools/index.html)

Service Line Material Inventory Andover has completed and submitted our service line materials inventory to the Minnesota Department of Health. The service line inventory is publicly available, and you can check the materials for your service line by visiting the Lead Inventory Tracking Tool (LITT) (https://maps.umn.edu/LSL/). You may also contact us at . To complete the service line inventory, our system . As of 05/03/2024, our inventory contains 0 lead, 0 galvanized requiring replacement, 0 unknown material, and 7196 non-lead service lines.

The Value of Water

Drinking water is a precious resource, yet we often take it for granted. Throughout history, civilizations have risen and fallen based on access to a plentiful, safe water supply. That’s still the case today. Water is key to healthy people and healthy communities.

Water is also vital to our economy. We need water for manufacturing, agriculture, energy production, and more. One-fifth of the U.S. economy would come to a stop without a reliable and clean source of water.

Systems are in place to provide you with safe drinking water. The state of Minnesota and local water systems work to protect drinking water sources. For example, we might work to seal an unused well to prevent contamination of the groundwater. We treat water to remove harmful contaminants. And we do extensive testing to ensure the safety of drinking water.

If we detect a problem, we take corrective action and notify the public. Water from a public water system like yours is tested more thoroughly and regulated more closely than water from any other source, including bottled water.

Conservation

Conservation is essential, even in the land of 10,000 lakes. For example, in parts of the metropolitan area, groundwater is being used faster than it can be replaced. Some agricultural regions in Minnesota are vulnerable to drought, which can affect crop yields and municipal water supplies. We must use our water wisely. Below are some tips to help you and your family conserve – and save money in the process.

Fix running toilets—they can waste hundreds of gallons of water.

Turn off the tap while shaving or brushing your teeth.

Shower instead of bathe. Bathing uses more water than showering, on average.

Only run full loads of laundry, and set the washing machine to the correct water level. Only run the dishwasher when it’s full.

Use water-efficient appliances (look for the WaterSense label).

Use water-friendly landscaping, such as native plants.

When you do water your yard, water slowly, deeply, and less frequently. Water early in the morning and close to the ground.

Learn more at Minnesota Pollution Control Agency’s Conserving Water webpage (https://www.pca.state.mn.us/living-green/conserving-water)

U.S. Environmental Protection Agency’s WaterSense webpage (https://www.epa.gov/watersense)

Home Water Treatment

Overview

Most Minnesotans, whether they drink from a public water supply or a private well, have drinking water that does not need treatment for health protection. Water treatment units are best for improving the physical qualities of water—the taste, color, or odor.

No single treatment process can remove all substances in water. If you decide to install a home water treatment unit, choose a unit certified and labeled to reduce or remove the substance of concern. If there is more than one substance you want to remove from your water, you may need to combine several treatment processes into one system.

Even well-designed treatments systems can fail. You should continue to test your drinking water after you install a treatment unit. All home water treatment units need regular maintenance to work correctly. Regular maintenance may include changing filters, disinfecting the unit, or cleaning scale buildup. Always install, clean, and maintain a treatment unit according to the manufacturer's recommendations.

Learn more at Home Water Treatment (https://www.health.state.mn.us/communities/environment/water/factsheet/hometreatment.html).

Beware of Water Treatment Scams

False claims, deceptive sales pitches, or scare tactics have been used by some water treatment companies. Every person has a right to decide what is best for themselves and their family, and you may choose to install additional water treatment to further lower the levels of contaminants of emerging concern, chlorine, and other chemicals in your water. However, you should be cautious about purchasing a water treatment system. If you are considering the purchase of a home water treatment system, please read the Minnesota Department of Health’s recommendations online at Warning: Beware of Water Treatment Scams (https://www.health.state.mn.us/communities/environment/water/factsheet/beware.html)

The Pros and Cons of Home Water Softening

When considering whether to use a water softener, contact your public water system to find out if you have hard water. Many systems treat for hardness, making water softeners unnecessary. Water softeners are a water treatment device. They remove water hardness (dissolved calcium and magnesium). Water softeners must be installed and maintained properly to be safe and effective. Learn more at Home Water Softening (https://www.health.state.mn.us/communities/environment/water/factsheet/softening.html).

The benefits of soft water include, Increased efficiency for soaps and detergents.Reduction in mineral staining on fixtures and in pipes. A potential increase in the lifespan of water heaters.

The drawbacks of soft water include: Operation and maintenance costs. More sodium. People on low-sodium diets should consult a doctor if they plan to regularly consume softened water. The production of salt brine as a byproduct. This can have negative effects at wastewater treatment plants and on ecosystems. Reduce the amount of salt brine used or install a salt-free system.